QUES: WHAT DO YOU MEAN BY DISPLAY DEVICES? WHAT ARE THE TYPES OF IT AND STATE ITS DAILY USES.

## What is a display?

In computing, a display is a device with a screen that shows a rendered electronic image made up of pixels that are illuminated in a way that distinguishes text and graphic elements. Pixels are tiny areas of illumination -- bright dots of light -- that can number in the millions on a single display. Today, most displays use colored pixels.

For displaying an image on the screen, the central processing unit (CPU) transfers the image data to another processor called a graphics processing unit (GPU). The GPU then interprets this data so that it can be understood by the display. What it does is divide this image into small colored points known as pixels. It therefore informs each pixel at specific positions on the screen about which color it should represent. This enables the display to illuminate appropriate pixels, thereby creating an entire image.

The working mechanisms behind the display are different from one another. They all depend on the display types. Even though, they have the same functionality. The engineering methods vary with each type of display. Following are the types:

TYPES OF DISPLAY DEVICE

- CRT (Cathode Ray Tube)

🡺 CRT is a vacuum tube display device.

🡺 It produces images by directing a focused

Electron beam onto a phosphorescent screen.

- When electrons strike the screen, it glows and forms images.

MAIN PARTS:

🡺 Electron gun: Produce and control electrons.

🡺 Deflection plates: Move the beam horizontally and vertically.

🡺 Phosphor screen: Emits light.

🡺 Glass envelope: Maintains vacuum.

WORKING (IN BRIEF):

🡺 Electron gun emits electrons →accelerated →deflected →hit screen →   image formed.

USES:

🡺 Old TVs and monitors

🡺 Cathode Ray Oscilloscope (CRO)

(2)  LCD (Liquid Crystal Display)

🡺 It is a flat panel display technology used in devices like Laptops, Monitors, TVs, Digital Watches, Calculator, etc.

🡺 An LCD works by using liquid crystals whose optical properties change when an electrical filed is applied

🡺 These crystals don’t emit light themselves they modulate light from a backlight (or reflector) to from images.

HOW LCD WORKS:

- The Backlight (often LED) provides light.

- Polarizing filters control light polarization.

- Liquid crystals rotate the light depending on applied voltage.

- Color filters create colors (Red, Blue, Black).

- Pixels are turned ON/OFF or dimmed from the visual image.

TYPES OF LCD:

🡺 Twisted Nematic (TN)

🡺 In-Plane Switching (IPS)

🡺 Vertical Alignment (VA)

ADVANTAGES OF LCD:

- Slim & Lightweight

- Low Power Consumption

- Good image quality.

DISADVANTAGES OF LCD:

- Limited Viewing Angles

- Response Time Issues

- Backlight dependency.

## (3) LED (Light Emitting Diode)

Type:
 LCD TV with LED backlight

Working:
 Uses light-emitting diodes placed behind or around the LCD panel to provide illumination. The LCD layer controls how much light passes through to form images.

Advantages:

- Energy efficient compared to traditional LCDs

- High brightness suitable for well-lit rooms

- Long lifespan with low heat generation

Disadvantages:

- Viewing angles are limited compared to OLED

- Black levels and contrast are not as deep as OLED

Daily Use:
 Televisions, computer monitors, laptops, digital boards, advertising displays

## (4) OLED (Organic Light Emitting Diode)

Type:
 Self-emissive display

Working:
 Each pixel is made of organic material that emits light when electricity flows through it, eliminating the need for a backlight.

Advantages:

- True black levels as pixels can turn completely off

- Very high contrast and vibrant colors

- Extremely thin, lightweight, and flexible design

Disadvantages:

- More expensive than LED displays

- Possibility of screen burn-in with prolonged static images

Daily Use:
 Smartphones, premium smart TVs, smartwatches, foldable and curved displays

## (5) Plasma Display

Type:
 Gas-based display technology

Working:
 Uses tiny gas-filled cells that become ionized when voltage is applied, producing ultraviolet light that excites phosphors to create visible images.

Advantages:

- Excellent color accuracy and natural picture quality

- Wide viewing angles with smooth motion display

Disadvantages:

- High power consumption and heat generation

- Heavy, bulky, and mostly outdated technology

Daily Use:
 Large-screen televisions (now mostly discontinued)

Sources:

- Geeksforgeeks

- Circuitgloal

- Techtarget

- Lenovo

- Wikipedia